For item 1 - please adjust the window just beside the edge of column. Window at 2F should aligned and adjusted as well.

Item 2 - rebar is extended at least 1m above 2F level where possible, others bent to the slab.

Item 3 - no beam on said location only additional horizontal on the top side of the shear wall below to compensate for the wall openings.

Item 4 - no beam.

Item 5 - it was designed to be a light steel structure. changing it would result to recalculation of the whole structure including foundations due to additional load.

For items 6, 7 & 10 - 0.30m from wall surface.

Item 8 - see drawing 2 on attached

Item 9 - see drawing 1 on attached

Item 11 - 0.60 from surface of wall of 0.65m from center line projected from 2F to Attic floor level.